Test Case

All Spine Mini Service Provider (SMSP) Host positive tests and SMS Client positive tests

Description

These tests are to prove:

- SMSP Host System – receive a valid request message and create and return a valid synchronous response.

- SMS Client System – create a valid request message and receive a response

Requirements Covered

All that relate to SMS positive messaging.

Applicability:

This test applies to:

- SMSP Host Applications

- SMS Client Applications

Test Execution

For the SMSP Host, run these tests using TKW Autotest Manager – – see TKW Autotest Manager Overview for details. For SMS Client, these tests are not currently supported in TKW Autotest Manager and will need to be run by the SUT sending a valid request message to TKW running Simulator mode – refer to Webcasts and/or TKW User Guide for more details.

Expected Results

Check the Automated Test Report produced from the Autotest run following the link to the Test Results Log that is produced on the Run Tests Tab after the test has been executed. This log should show a Pass against all the tests within the test case.

Check the validation report by following the link on the Automated Test Report, check that the validation reports for this test case have all passed validation.

SMSP Host Application Autotest Manager Tests

Autotest Manager will transmit a valid request message for the selected tests to the System Under Test. It expects a valid response to be returned from the System Under Test, and when received it will execute the following tests:

Test 1	Ensure that the response is a HTTP 200 Response

Test2	Ensures that the Action Element in the SOAP Header of the response message is equal to the value of the input message suffixed with Response e.g. urn:nhs-itk:services:201005:getPatientDetails-v1-0Response

Test3	If the WS-Security elements are included in the response (Requirement WS-SEC-16), it ensures that the Created Timestamp Element in the SOAP Header of the response message is not less than the Created Timestamp Element in the input message

Test4  	Ensures that the MessageID in the SOAP Header of the response message is not equal to the MessageID in the SOAP Header of the input message

SMS Client Tests

SUT to send a message to TKW running in Simulator mode. TKW Simulator will respond with a valid response message. The message interaction will be captured in the simulator_saved_messages folder in the TKW Config folder, submit this log as evidence.