COMMERCIAL - IN CONFIDENCE
	National SPINE
	
External Interface Specification: Part 3 Message Interaction Map
	Issue 12.3
24/4/2015





	NHS CFH National SPINE
Requirements, Design and Delivery


External Interface Specification: Part 3 Message Interaction Map
Issue 12.3
24/04/2015





Contents
43
Message Interaction Map


43.1
Overview


43.2
Changes


53.3
Table Keys


53.3.1
Statuses


73.3.2
Application Acknowledgements


73.4
Domain Versions


103.5
Spine Non HL7 Services


103.5.1
Version 1.0


103.6
EBS (AKA Choose and Book)


103.6.1
Version 2.1


123.6.2
Version 3.5


153.7
GP2GP


153.7.1
Version 1.9


163.7.2
Version 2.2


163.7.3
Version 2.3


173.8
LRS


173.8.1
Version CRS-1.0


193.8.2
Version 2.7


193.8.3
Version 4.4


213.9
Medication Management


213.9.1
Version 1.4


233.9.2
Version 2.3.1


263.10
PDS


263.10.1
Version 1.1


283.10.2
Version 2.8


313.10.3
Version 3.2


333.10.4
Version 4.3


373.10.5
Version 5.2


403.11
GP Summary


403.11.1
Version 2.8


413.12
PSIS Query


413.12.1
Version 1.7


423.12.2
Version 2.6


433.13
Discharge


433.13.1
Version 3.6


443.14
Emergency Department


443.14.1
Version 3.6


463.15
OutPatients


463.15.1
Version 1.6


473.16
Alerts


473.16.1
Version 1.1


473.17
CAF (AKA Health and Social Care Integration)


473.17.1
Version TBD-1.0


483.17.2
Version 1.0


503.17.3
Version 2.1


533.17.4
Version 2.2


603.18
Pathology (AKA Laboratory)


603.18.1
Version 1.1


613.19
Common Content (also known as Common Point to Point)


613.19.1
Version 1.1


623.20
Admissions


623.20.1
Version 1.1


633.21
Out of Hours


633.21.1
Version 1.0


643.22
Ambulance Messaging


643.22.1
Version 1.1


653.23
Mental Health


653.23.1
Version 1.0


703.24
NHS Direct


703.24.1
Version 1.0


713.25
Diagnostic Imaging Reporting


713.25.1
Version 1.0


723.26
General Practice Extraction Service


723.26.1
Version 1.0


743.27
Inter-operability Toolkit Service


743.27.1
Version 1.0


743.28
Child Protection Information Service


743.28.1
Version 1.0




3 Message Interaction Map
Overview
This section associates HL7/Spine operational forms and message behaviour patterns, described in this specification, with HL7 messages defined in the MIM. Implementations MUST support the MHS, HL7 and Spine non-HL7 behaviours defined. The details of the interface behaviour (the contract properties used in ebXML) are configured in the SDS, and no longer a normative part of this specification. 
 Each subsection defines the behavioural elements for each MIM/Spine message domain implemented in Release 2008-B. For each domain the following is provided: 
· HL7/Spine non-HL7 Interaction. A message exchange begins with a specific interaction and a responding interaction. Some domains also require responses to responses. The interaction is described in its colloquial and normative form. The normative form is of the xxxx_INnnnnnnUKvv form for HL7 interactions - Spine interactions are defined here and elsewhere within the EIS.
· MHS Message behaviour. This is the specific configured behaviour documented for MHS protocol implementations. The behaviour is described in the external document [BT-Mbeh]. MHS implementations use [BT-Mbeh] (as well as the MHS section of this document) to specialise protocol behaviour left otherwise unconstrained by industry standard protocols (such as ebXML and web services). The following behaviours are adopted from [BT-Mbeh] to the message domains:
· Web Service
· ebXML Reliable Asynchronous
· Multi-hop End-Party Reliability
· Multi-hop Intermediary Reliability
· Unreliable Asynchronous
· No-Party Reliability
· HL7 Forms of Operation. The HL7 behavioural forms describe how the layer above (the HL7 processor) behaves from a protocol perspective. This is documented in Part 2 of this specification. The two options supported by this specification are RIF Form and State Exchange Form.
· Error codes and messages have been removed from the MIM so that they may be described by each service owner. At this point the domain error codes for services owned by BT are documented in this specification.
Changes
 This section of the EIS has undergone some changes since the 2006-B-2 release - these include: 
· The inclusion of 'status' against interactions/domains to provide readers with an insight into the author's view of the current status/stability of the specific interaction and message domain.
· The inclusion of non-HL7 interactions
· The inclusion of all application acknowledgement interactions that are technically necessary (including those sent by the Spine MHS). Note that these may not be referenced in the MiM.
· Inclusion of 'P1R1' interactions such that this section provides a holistic view of all interactions supported by the Spine: previously these were defined by [EIS 5.5]. Note that behaviour for these messages is still described by EIS 5.5 and is included here for ease of reference.
· Web service message response wrappers (as defined in WSDL) are indicated where pertinent.
Table Keys
 The tables below provide keys for the information contained within this part of the EIS. 
Statuses
	Key
	Description
	Further information

	draft
	The interaction (or interactions within the domain) has been fully elaborated and included in draft documentation/MiMs.
	The interaction (or interactions within the domain) is likely to be relatively stable; however the interaction is under review and may change as a result of review comments. Some draft interactions MAY still be configured on Spine.

	issued
	The interaction (or interactions within the domain) has been fully elaborated, reviewed by most parties and published in official documentation/MiM.
	The interaction (or interactions within the domain) is likely to be stable; however the interaction is could change as a result of review comments and or implementation issues.

	new
	The interaction (or interactions within the domain) is normative and is new for this release.
	The interaction (or interactions within the domain) is stable and is accepted by all parties and will be supported by the current Spine release.

	existing
	The interaction (or interactions within the domain) is normative and was defined as part of a previous release.
	The interaction (or interactions within the domain) is stable and is accepted by all parties. The interaction support was defined by a previous Spine release; the support will be maintained by this Spine release.

	on-hold
	The interaction (or interactions within the domain) has been put on hold because - although published - its use has been forbidden in the MiM or has been postponed until a future release or because its use is pending further discussion between stakeholders.
	Information about the interaction is provided for reference only: the interaction should not be used whilst it is in the 'on-hold' state.

	rejected
	The interaction (or interactions within the domain) has been rejected and should not be used.
	Information about the rejected interaction is provided for reference only: the interaction should not be used.

	deprecated
	The interaction (or interactions within the domain) has been deprecated and MUST not be used by new implementations.
	The interaction (or interactions within the domain) will still be supported in the live environment for the current release; however subsequent release may remove support for this interaction. Implementers MUST use the appropriate updated interactions in its stead.

	retired
	The interaction (or interactions within the domain) has been retired and will no longer be supported by this or subsequent releases.
	Implementers should use the appropriate updated interactions instead of the retired interaction.

	superseded
	The domain version has been superseded.
	The domain version has been superseded, however all the interactions in the domain are still supported by the domain version that supersedes it. Note that in any responses Spine will EITHER reflect the versionCode of the HL7 request OR will return the versionCode of the latest published MIM in which that interaction appears.

	Mixed
	When referring to the status of a message domain, this indicates that not all the interactions within the domain are in the same state.
	Implementers should understand the details of the status of the individual interactions within the domain to understand the overall status of the domain.


Application Acknowledgements
Application Acknowledgments may be sent from the MHS as well as from the endpoint. Sometimes they will be sent by both. The indicator below signals in what context an MHS should expect an Application Acknowledgement for an interaction.
	Key
	Description

	Nothing indicated
	The application acknowledgement may be sent by both the end application as well as the Spine MHS.

	†
	The application acknowledgement will only be sent from the end application.

	*
	The application acknowledgement will only be sent from the Spine MHS.

	‡
	The Spine MHS may send this standard end application response minus the payload as a 'pseudo' application acknowledgement - see [BT-Mbeh] or section 2.5.4 of [EIS 5.5] for more details.


Domain Versions
The following table defines the MIM and domain versions supported by the 2008-B release. Messages defined in [EIS9.5] are superseded by this list.
	Domain
	Domain Version
	Associated MiM Version
	Status

	Spine Non HL7 Services
	1.0
	n/a
	Existing

	EBS
	2.1
	[V3NPfIT2.3]
	Existing

	
	3.5
	[V3NPfIT3.1.09]
	Existing

	
	4.1
	[V3NPfIT4.2.00]
	superseded by 4.2

	
	4.2
	[V3NPfIT6.2.02]
	Retired

	
	4.3
	[V3NPfIT7.2.02]
	Retired

	GP2GP
	1.5
	[V3NPfIT3.1.07]
	Retired

	
	1.8
	[V3NPfIT3.1.10]
	superseded by 1.9

	
	1.9
	[V3NPfIT4.2.00]
	existing

	
	2.2
	[V3NPfIT6.2.02]
	existing

	
	2.3
	[V3NPfIT6.2.02]
	New combination (existing interaction on an existing service)

	Infrastructure
	2.1
	[V3NPfIT2.3]
	existing

	
	3.5
	[V3NPfIT3.1.07]
	existing

	
	3.7
	[V3NPfIT3.1.09]
	existing

	
	3.8
	[V3NPfIT3.1.10]
	existing

	
	3.9
	[V3NPfIT4.1.00]
	existing

	
	3.12
	[V3NPfIT4.2.00]
	retired

	
	5.5
	[V3NPfIT6.2.02]
	existing and on-hold

	
	5.6
	[V3NPfIT7.2.02]
	retired

	LRS
	CRS-1.0
	n/a
	existing

	
	1.0
	[V3NPfIT2.2]
	retired

	
	2.7
	[V3NPfIT3.1.10]
	existing

	
	4.4
	[V3NPfIT6.2.02]
	mixed: existing and issued

	Medication Management
	1.4
	[V3NPfIT3.1.07]
	existing

	
	2.1
	[V3NPfIT4.1.00]
	superseded by 2.3.1

	
	2.3.1
	[V3NPfIT4.2.00]
	existing

	
	3.1.3
	[V3NPfIT6.2.02]
	retired

	PDS
	1.1
	[V3NPfIT2.3]
	deprecated

	
	2.7
	[V3NPfIT3.1.10]
	superseded by 2.8

	
	2.8
	[V3NPfIT3.1.11]
	existing

	
	3.2
	[V3NPfIT4.2.00]
	existing

	
	4.3
	[V3NPfIT6.2.02]
	existing

	
	5.2
	[V3NPfIT7.2.02]
	mixed: existing and new

	GP Summary
	2.6
	[V3NPfIT4.2.00]
	superseded by 2.8

	
	2.8
	[V3NPfIT6.2.02]
	existing

	PSIS Query
	CRS-1.0
	n/a
	retired

	
	1.7
	[V3NPfIT4.2.00]
	existing

	
	2.4
	[V3NPfIT6.2.02]
	retired

	
	2.5
	[V3NPfIT6.3.01]
	superseded by 2.6

	
	2.6
	[V3NPfIT7.2.02]
	new

	Discharge
	3.4
	[V3NPfIT6.2.02]
	retired

	
	3.6
	[V3NPfIT6.3.01]
	superseded by 3.6

	
	3.6
	[V3NPfIT7.2.02]
	new

	Emergency Department
	3.4
	[V3NPfIT6.2.02]
	retired

	
	3.6
	[V3NPfIT6.3.01]
	superseded by 3.6

	
	3.6
	[V3NPfIT7.2.02]
	new

	HealthSpace
	1.3
	[V3NPfIT6.2.02]
	retired

	
	1.5
	[V3NPfIT6.3.01]
	retired

	
	1.5
	[V3NPfIT7.2.02]
	retired

	OutPatients
	1.4
	[V3NPfIT6.2.02]
	retired

	
	1.6
	[V3NPfIT6.3.01]
	superseded by 1.6

	
	1.6
	[V3NPfIT7.2.02]
	new

	Sealed Envelopes
	1.3
	[V3NPfIT6.2.02]
	retired

	
	1.5
	[V3NPfIT6.3.01]
	retired

	
	1.5
	[V3NPfIT7.2.02]
	retired

	Alerts
	1.0
	[V3NPfIT6.2.02]
	superseded by 1.1

	
	1.1
	[V3NPfIT7.2.02]
	new

	CAF (also known as Health and Social Care Integration)
	TBD-1.0
	n/a
	existing

	
	1.0
	[V3NPfIT6.2.02]
	existing

	
	2.1
	[V3NPfIT7.2.02]
	mixed: existing and new

	
	2.2
	[HSCI DMIM Version 3 Draft 07/01/2011]
	new

	Pathology
	1.1
	[V3NPfIT6.2.02]
	existing

	Common Content (also known as Common Point to Point)
	1.0
	[V3NPfIT6.2.02]
	superseded by 1.1

	
	1.1
	[V3NPfIT7.2.02]
	new

	Admissions
	1.1
	[V3NPfIT7.2.02]
	new

	Out of Hours
	1.1
	[V3NPfIT7.2.02]
	new

	Ambulance Messaging
	1.1
	[V3NPfIT7.2.02]
	new

	Mental Health
	1.0
	[V3NPfIT7.2.02]
	new

	NHS Direct
	1.0
	[V3NPfIT7.2.02]
	new

	Diagnostic Imaging Reporting
	1.0
	[V3NPfIT7.2.02]
	New

	General Practice Extraction Service
	1.0
	[GPES DMS v1.0 RC1]
	New

	Interoperability Toolkit Service
	1.0
	[GPES DMS v1.0 RC1]
	New


Spine Non HL7 Services
Spine Non HL7 Services interactions exist under the services urn:nhs:names:services:sdsquery, urn:nhs:names:services:lmquery and urn:nhs:names:services:lm
Version 1.0
Service: urn:nhs:names:services:sdsquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	QASI: Query Accredited System Information
Request:
· QASI: Query Accredited System Information (AccreditedSystemSearchRequest_1_0)
Response:
· QASI: Accredited System Information Response (AccreditedSystemSearchResponse_1_0)
	Web Service
	N/A
	existing


EBS (AKA Choose and Book)
EBS interactions exist under the services urn:nhs:names:services:ebs, urn:nhs:names:services:ebsnpr and urn:nhs:names:services:ebsepr
The EBS service will support [MIM2.3]; [MIM3.1.09] and [MIM7.2]
Note that EBS HL7 behaviour is not described by the HL7 Forms described in the EIS. EBS HL7 behaviour can be obtained from the EBS service provider's compliance information.
Version 2.1
Service: urn:nhs:names:services:ebs
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Request Service
Request:
· Request Service (PRPA_IN010000UK02)
Response:
· Confirm Service Request (PRPA_IN020000UK02)
Response to Response:
· Application Acknowledgement (MCCI_IN010000UK01)†
	Multi-Hop End-Party Reliability
	N/A
	existing

	Request Appointment Confirmation
Request:
· Request Appointment Confirmation (PRSC_IN040000UK03)
Response:
· Confirm Appointment (PRSC_IN050000UK02)
· Reject Appointment (PRSC_IN060000UK02)
Response to Response:
· Application Acknowledgement (MCCI_IN010000UK01)†
	Multi-Hop End-Party Reliability
	N/A
	existing

	Request Appointment Cancellation
Request:
· Request Appointment Cancellation (PRSC_IN070000UK03)
Response:
· Confirm Appointment Cancellation (PRSC_IN140000UK02)
· Reject Appointment Cancellation (PRSC_IN150000UK02)
Response to Response:
· Application Acknowledgement (MCCI_IN010000UK01)†
	Multi-Hop End-Party Reliability
	N/A
	existing

	Notify Appointment Cancellation
Request:
· Notify Appointment Cancellation (PRSC_IN130000UK02)
Response:
· Application Acknowledgement (MCCI_IN010000UK01)†
	Multi-Hop End-Party Reliability
	N/A
	existing

	Request Available Slots
Request:
· Request Available Slots (PRSC_IN080000UK03)
Response:
· Respond Available Slots (PRSC_IN090000UK03)
· Application Acknowledgement (MCCI_IN010000UK01)†
Response to Response:
· Application Acknowledgement (MCCI_IN010000UK01)†
	Multi-Hop End-Party Reliability
	N/A
	existing

	Notify Slot Cancellation
Request:
· Notify Slot Cancellation (PRSC_IN100000UK02)
Response:
· Application Acknowledgement (MCCI_IN010000UK01)†
	Multi-Hop End-Party Reliability
	N/A
	existing

	Notify Appointment Did Not Attends
Request:
· Notify Appointment Did Not Attends (PRSC_IN110000UK03)
Response:
· Application Acknowledgement (MCCI_IN010000UK01)†
	Multi-Hop End-Party Reliability
	N/A
	existing


Version 3.5
Service: urn:nhs:names:services:ebs
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Request Service
Request:
· Request Service (PRPA_IN010000UK07)
Response:
· Confirm Service Request (PRPA_IN020000UK06)
· Application Acknowledgement (MCCI_IN010000UK13)*
Response to Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	N/A
	existing

	Request Appointment Confirmation
Request:
· Request Appointment Confirmation (PRSC_IN040000UK08)
Response:
· Confirm Appointment (PRSC_IN050000UK06)
· Reject Appointment (PRSC_IN060000UK06)
· Application Acknowledgement (MCCI_IN010000UK13)*
Response to Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	N/A
	existing

	Request Appointment Cancellation
Request:
· Request Appointment Cancellation (PRSC_IN070000UK08)
Response:
· Confirm Appointment Cancellation (PRSC_IN140000UK06)
· Reject Appointment Cancellation (PRSC_IN150000UK06)
· Application Acknowledgement (MCCI_IN010000UK13)*
Response to Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	N/A
	existing

	Notify Appointment Cancellation
Request:
· Notify Appointment Cancellation (PRSC_IN130000UK07)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	N/A
	existing

	Request Available Slots
Request:
· Request Available Slots (PRSC_IN080000UK07)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· Respond Available Slots (PRSC_IN090000UK09)
Response to Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	N/A
	existing

	Notify Slot Cancellation
Request:
· Notify Slot Cancellation (PRSC_IN100000UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	N/A
	existing

	Notify Appointment Did Not Attends
Request:
· Notify Appointment Did Not Attends (PRSC_IN110000UK08)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	N/A
	existing

	Notify Patient Referral
Request:
· Notify Patient Referral (PRPA_IN030000UK08)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	N/A
	existing


GP2GP
GP2GP interactions all exist under the service urn:nhs:names:services:gp2gp
Version 1.9
Service: urn:nhs:names:services:gp2gp
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Electronic Healthcare Record Request Started
Request:
· Electronic Healthcare Record Request Started (RCMR_IN010000UK05)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Electronic Healthcare Record Request Completed
Request:
· Electronic Healthcare Record Request Completed (RCMR_IN030000UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing


Version 2.2
Service: urn:nhs:names:services:gp2gp
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Electronic Healthcare Record Request Started
Request:
· Electronic Healthcare Record Request Started (RCMR_IN010000UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Electronic Healthcare Record Request Completed
Request:
· Electronic Healthcare Record Request Completed (RCMR_IN030000UK08)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing


Version 2.3

Service: urn:nhs:names:services:gp2gp

	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Common Content Forward Reliable
Request:
· Common Content Forward Reliable (COPC_IN000001UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	New combination (Existing interaction on and additional existing service) 


LRS
LRS interactions exist under the services urn:nhs:names:services:lrs, urn:nhs:names:services:lrsquery, urn:nhs:names:services:sdsquery and urn:nhs:names:services:sds
LRS actions defined under the service urn:nhs:names:services:lrsquery MUST be accepted under service lrs or lrsquery until the delivery of 2008-B: when sending request actions on the ebXML channel (e.g. QUPA_IN020000UK31) the receiver MUST return a business response under the same service as received in the request (either lrs or lrsquery). However, the business error signal (MCCI_IN010000UK13) MUST be sent under the lrsquery service. Receivers of the business error signal (MCCI_IN010000UK13) MUST have an MHS object registered in SDS with contract properties equal to the Unreliable Asynchronous Message Pattern defined in [BT-Mbeh], under the lrsquery service. This behaviour is REQUIRED from 2006-A onwards. Note that existing clients submitting queries under service lrs are being supported during 2008-A for the purposes of transition.
For error codes returned by LRS see Part B of this specification.
  Note:  LRS 1.0 domain version support is withdrawn in this release.
Version CRS-1.0
Service: urn:nhs:names:services:lrs
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Set Resource Permissions
Request:
· Set Resource Permissions (SET_RESOURCE_PERMISSIONS_INUK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
Note:  Responses to this query will be returned in an XML wrapper called setResourcePermissionsResponse.
	Web Service
	RIF Form
	existing


Service: urn:nhs:names:services:lrsquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Get Resource Permissions
Request:
· Get Resource Permissions (GET_RESOURCE_PERMISSIONS_INUK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Get Resource Permissions Response (GET_RESOURCE_PERMISSIONS_RESPONSE_INUK01)
Note:  Responses to this query will be returned in an XML wrapper called getResourcePermissionsResponse.
	Web Service
	RIF Form
	existing

	Has Resource Permissions
Request:
· Has Resource Permissions (HAS_RESOURCE_PERMISSIONS_INUK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Has Resource Permissions Response (HAS_RESOURCE_PERMISSIONS_RESPONSE_INUK01)
Note:  Responses to this query will be returned in an XML wrapper called hasResourcePermissionsResponse.
	Web Service
	RIF Form
	existing


Version 2.7
Service: urn:nhs:names:services:lrs
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Request Create LR
Request:
· Request Create LR (REPC_IN010000UK15)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· Confirm LR created (REPC_IN020000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Request Status Change
Request:
· Request Status Change (REPC_IN040000UK15)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· Confirm Status Change Request (REPC_IN050000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing


Version 4.4
Service: urn:nhs:names:services:lrs
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Request Create LR
Request:
· Request Create LR (REPC_IN010000UK15)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· Confirm LR created (REPC_IN020000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Request Status Change
Request:
· Request Status Change (REPC_IN040000UK15)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· Confirm Status Change Request (REPC_IN050000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing


Service: urn:nhs:names:services:lrsquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Query Confirmation Request
Request:
· Query Confirmation Request (QUPC_IN010000UK32)
Response:
· Query Act Failed (QUQI_IN010000UK14)
· Confirm LR Short Response (QUPC_IN030000UK14)
· Confirm LR - History Response (QUPC_IN040000UK30)
· Application Acknowledgement (MCCI_IN010000UK13)*
Note:  Responses to this query will be returned in an XML wrapper called queryConfirmationResponse.
	Web Service
	RIF Form
	issued

	Query Details
Request:
· Query Details (QUPC_IN020000UK17)
Response:
· Query Act Failed (QUQI_IN010000UK14)
· LR Details - Simple Response (QUPC_IN060000UK32)
· LR Details - Complete Response (QUPC_IN070000UK32)
· Application Acknowledgement (MCCI_IN010000UK13)*
	ebXML Unreliable Asynchronous
	RIF Form
	issued


Medication Management
Medication Management interactions exist under two services urn:nhs:names:services:mm and urn:nhs:names:services:mmquery
For error codes returned by ETP see Part B of this specification.
ETPv2 needs to be implemented in distinct phases to ensure that invalid message combinations are not encountered. Detailed information regarding compatibility between ETP versions is available in [ETP-SS].
Version 1.4
Service: urn:nhs:names:services:mm
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Parent Prescription
Request:
· Parent Prescription (PORX_IN020101UK04)
Response:
· Application Acknowledgement (MCCI_IN010000UK12)*
· Parent Prescription Rejection (PORX_IN250101UK04)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Parent Prescription (Non-urgent)
Request:
· Parent Prescription (Non-urgent) (PORX_IN020102UK04)
Response:
· Application Acknowledgement (MCCI_IN010000UK12)*
· Parent Prescription Rejection (PORX_IN250101UK04)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Patient Prescription Release Request
Request:
· Patient Prescription Release Request (PORX_IN132004UK04)
Response:
· Application Acknowledgement (MCCI_IN010000UK12)*
· Patient Prescription Release Response (PORX_IN070103UK04)
· Prescription Release Rejection (PORX_IN110101UK04)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Dispense Notification
Request:
· Dispense Notification (PORX_IN080101UK04)
Response:
· Application Acknowledgement (MCCI_IN010000UK12)*
· Dispense Notification Rejection (PORX_IN260101UK04)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Dispense Proposal Return
Request:
· Dispense Proposal Return (PORX_IN100101UK04)
Response:
· Application Acknowledgement (MCCI_IN010000UK12)*
· Dispense Proposal Return Rejection (PORX_IN240101UK04)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing


Version 2.3.1
Service: urn:nhs:names:services:mm
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Parent Prescription
Request:
· Parent Prescription (PORX_IN020101UK31)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Parent Prescription (Non-urgent)
Request:
· Parent Prescription (Non-urgent) (PORX_IN020102UK31)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Patient Prescription Release Request
Request:
· Patient Prescription Release Request (PORX_IN132004UK30)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· Patient Prescription Release Response (PORX_IN070103UK31)
· Prescription Release Rejection (PORX_IN110101UK30)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Nominated Prescription Release Request
Request:
· Nominated Prescription Release Request (PORX_IN060102UK30)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· Nominated Prescription Release Response (PORX_IN070101UK31)
· Prescription Release Rejection (PORX_IN110101UK30)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Dispense Proposal Return
Request:
· Dispense Proposal Return (PORX_IN100101UK31)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Cancel Request
Request:
· Cancel Request (PORX_IN030101UK32)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· Cancel Response (PORX_IN050101UK31)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Subsequent Cancel Response
Request:
· Subsequent Cancel Response (PORX_IN050102UK32)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Dispense Notification
Request:
· Dispense Notification (PORX_IN080101UK31)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Dispense Claim Information
Request:
· Dispense Claim Information (PORX_IN090101UK31)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· Dispense Reimbursement Claim Reject (PORX_IN150101UK31)
  Note:  The Reimbursement Claim Reject (generated by the Payment Authority and passed to Dispenser via the Spine) is modelled as a response to an original claim submission (or resubmission). It is exchanged in this manner because the business error is carried in the Control Act of the response. The HL7 messageRef/id/@root will contain the UUID of the original request message that triggered ETP to send the claim to the Payment Authority. Additionally, the ebXML MessageData/RefToMessageId will be absent.
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Dispenser Withdraw
Request:
· Dispenser Withdraw (PORX_IN510101UK31)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Rebuild Dispense History
Request:
· Rebuild Dispense History (PORX_IN540101UK31)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing


PDS
PDS interactions exist under two services urn:nhs:names:services:pds and urn:nhs:names:services:pdsquery
PDS actions defined under the service urn:nhs:names:services:pdsquery MUST be accepted under service pds or pdsquery until the delivery of 2008-B: when sending request actions on the ebXML channel (e.g. QUPA_IN020000UK31) the receiver MUST return a business response under the same service as received in the request (either pds or pdsquery). However, the business error signal (MCCI_IN010000UK13) MUST be sent under the pdsquery service. Receivers of the business error signal (MCCI_IN010000UK13) MUST have an MHS object registered in SDS with contract properties equal to the Unreliable Asynchronous Message Pattern defined in [BT-Mbeh], under the pdsquery service. This behaviour is REQUIRED from 2006-A onwards. Note that existing clients submitting queries under service pds are being supported during 2008-A for the purposes of transition.
For error codes returned by PDS see Part B of this specification.
Version 1.1
Service: urn:nhs:names:services:pds
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PDS Trace Query Started
Request:
· PDS Trace Query Started (QUPA_IN010000UK01)
Response:
· Query Act Failed (QUQI_IN010000UK01)‡
· PDS Trace Query Successful (QUPA_IN030000UK02)
	Web Service
	RIF Form
	deprecated

	PDS Advanced Trace Query Started
Request:
· PDS Advanced Trace Query Started (QUPA_IN020000UK01)
Response:
· Query Act Failed (QUQI_IN010000UK01)‡
· PDS Trace Query Successful (QUPA_IN030000UK02)
	ebXML Reliable Asynchronous
	State Exchange Form
	deprecated

	PDS Retrieval Query Started
Request:
· PDS Retrieval Query Started (QUPA_IN040000UK01)
Response:
· Query Act Failed (QUQI_IN010000UK01)‡
· PDS Retrieval Query Successful (QUPA_IN050000UK01)
	Web Service
	RIF Form
	deprecated

	PDS Death Update Started
Request:
· PDS Death Update Started (PRPA_IN100000UK01)
Response:
· PDS Update Failure Response (PRPA_IN140000UK01)‡
· PDS Successful Update Response (PRPA_IN150000UK01)
	ebXML Reliable Asynchronous
	State Exchange Form
	deprecated

	PDS General Update Started
Request:
· PDS General Update Started (PRPA_IN110000UK01)
Response:
· PDS Update Failure Response (PRPA_IN140000UK01)‡
· PDS Successful Update Response (PRPA_IN150000UK01)
	ebXML Reliable Asynchronous
	State Exchange Form
	deprecated

	PDS Consent Update Started
Request:
· PDS Consent Update Started (PRPA_IN120000UK01)
Response:
· PDS Update Failure Response (PRPA_IN140000UK01)‡
· PDS Successful Update Response (PRPA_IN150000UK01)
	ebXML Reliable Asynchronous
	State Exchange Form
	deprecated


Version 2.8
Service: urn:nhs:names:services:pdsquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PDS Trace Query Started
Request:
· PDS Trace Query Started (QUPA_IN010000UK13)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Trace Query Successful (QUPA_IN030000UK15)
Note:  Responses to this query will be returned in an XML wrapper called traceQueryResponse.
	Web Service
	RIF Form
	existing

	PDS Advanced Trace Query Started
Request:
· PDS Advanced Trace Query Started (QUPA_IN020000UK14)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Trace Query Successful (QUPA_IN030000UK15)
	ebXML Unreliable Asynchronous
	RIF Form
	existing

	PDS Retrieval Query Started
Request:
· PDS Retrieval Query Started (QUPA_IN040000UK14)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Retrieval Query Successful (QUPA_IN050000UK16)
Note:  Responses to this query will be returned in an XML wrapper called retrievalQueryResponse.
	Web Service
	RIF Form
	existing

	PDS Confirm NHS Number Query Started
Request:
· PDS Confirm NHS Number Query Started (QUPA_IN060000UK13)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS NHS Number Confirmed (QUPA_IN070000UK16)
Note:  Responses to this query will be returned in an XML wrapper called confirmNHSResponse.
	Web Service
	RIF Form
	existing


Service: urn:nhs:names:services:pds
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PDS NHS Number Allocation Request Started
Request:
· PDS NHS Number Allocation Request Started (PRPA_IN040000UK15)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS NHS Number Allocation Request Completed (PRPA_IN060000UK14)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	PDS Death Update Started
Request:
· PDS Death Update Started (PRPA_IN100000UK14)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS Successful Update Response (PRPA_IN150000UK14)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	PDS General Update Started
Request:
· PDS General Update Started (PRPA_IN110000UK15)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS Successful Update Response (PRPA_IN150000UK14)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	PDS Consent Update Started
Request:
· PDS Consent Update Started (PRPA_IN120000UK14)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS Successful Update Response (PRPA_IN150000UK14)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	PDS Combined Update Started
Request:
· PDS Combined Update Started (PRPA_IN160000UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS Successful Update Response (PRPA_IN150000UK14)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing


Version 3.2
Service: urn:nhs:names:services:pdsquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PDS Trace Query Started
Request:
· PDS Trace Query Started (QUPA_IN010000UK32)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Trace Query Successful (QUPA_IN030000UK32)
Note:  Responses to this query will be returned in an XML wrapper called traceQueryResponse.
	Web Service
	RIF Form
	existing

	PDS Advanced Trace Query Started
Request:
· PDS Advanced Trace Query Started (QUPA_IN020000UK31)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Trace Query Successful (QUPA_IN030000UK32)
	ebXML Unreliable Asynchronous
	RIF Form
	existing

	PDS Retrieval Query Started
Request:
· PDS Retrieval Query Started (QUPA_IN040000UK32)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Retrieval Query Successful (QUPA_IN050000UK32)
Note:  Responses to this query will be returned in an XML wrapper called retrievalQueryResponse.
	Web Service
	RIF Form
	existing

	PDS Confirm NHS Number Query Started
Request:
· PDS Confirm NHS Number Query Started (QUPA_IN060000UK30)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS NHS Number Confirmed (QUPA_IN070000UK30)
Note:  Responses to this query will be returned in an XML wrapper called confirmNHSResponse.
	Web Service
	RIF Form
	existing


Service: urn:nhs:names:services:pds
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PDS NHS Number Allocation Request Started
Request:
· PDS NHS Number Allocation Request Started (PRPA_IN040000UK30)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS NHS Number Allocation Request Completed (PRPA_IN060000UK30)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	General Update Started
Request:
· General Update Started (PRPA_IN160000UK30)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS Successful Update Response (PRPA_IN150000UK30)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing


Version 4.3
Service: urn:nhs:names:services:pdsquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PDS Simple Trace Query
Request:
· PDS Simple Trace Query (QUPA_IN000005UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Simple Trace Query Response (QUPA_IN000007UK01)
Note:  Responses to this query will be returned in an XML wrapper called traceQueryResponse.
	Web Service
	RIF Form
	existing

	PDS Advanced Trace Query
Request:
· PDS Advanced Trace Query (QUPA_IN000006UK02)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Advanced Trace Query Response (QUPA_IN000011UK02)
	ebXML Unreliable Asynchronous
	RIF Form
	existing

	PDS Retrieval Query
Request:
· PDS Retrieval Query (QUPA_IN000008UK02)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Retrieval Query Response (QUPA_IN000009UK03)
Note:  Responses to this query will be returned in an XML wrapper called retrievalQueryResponse.
	Web Service
	RIF Form
	existing

	PDS Confirm NHS Number Query
Request:
· PDS Confirm NHS Number Query (QUPA_IN000016UK02)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Confirm NHS Number Query Response (QUPA_IN000010UK02)
Note:  Responses to this query will be returned in an XML wrapper called confirmNHSNumberResponse.
	Web Service
	RIF Form
	existing

	PDS Cross Check Trace Query
Request:
· PDS Cross Check Trace Query (QUPA_IN000014UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Cross Check Trace Query Response (QUPA_IN000015UK02)
Note:  Responses to this query will be returned in an XML wrapper called crossCheckTraceQueryResponse.
	Web Service
	RIF Form
	existing


Service: urn:nhs:names:services:pds
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PDS NHS Number Allocation Request
Request:
· PDS NHS Number Allocation Request (PRPA_IN000200UK03)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS NHS Number Allocation Request Accepted (PRPA_IN000201UK01)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	PDS General Update Request
Request:
· PDS General Update Request (PRPA_IN000203UK03)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS General Update Request Accepted (PRPA_IN000202UK01)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	PDS NHAIS Update Request
Request:
· PDS NHAIS Update Request (PRPA_IN000204UK03)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS NHAIS Update Request Accepted (PRPA_IN000209UK01)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	PDS Create Initial Record Request
Request:
· PDS Create Initial Record Request (PRPA_IN000205UK03)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS Create Initial Record Request Rejected (Duplicate Found) (PRPA_IN000207UK03)
· PDS Create Initial Record Request Accepted (No Duplicate Found) (PRPA_IN000206UK01)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	PDS Birth Notification
Request:
· PDS Birth Notification (PRPA_IN000208UK03)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing


Version 5.2
Service: urn:nhs:names:services:pds
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PDS General Update Request
Request:
· PDS General Update Request (PRPA_IN000203UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS General Update Request Accepted (PRPA_IN000202UK01)
	ebXML Reliable Asynchronous
	State Exchange Form
	new

	PDS Create Initial Record Request
Request:
· PDS Create Initial Record Request (PRPA_IN000205UK05)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
· PDS Create Initial Record Request Rejected (Duplicate Found) (PRPA_IN000207UK05)
· PDS Create Initial Record Request Accepted (No Duplicate Found) (PRPA_IN000206UK01)
	ebXML Reliable Asynchronous
	State Exchange Form
	new

	PDS Birth Notification
Request:
· PDS Birth Notification (PRPA_IN000208UK03)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing


Service: urn:nhs:names:services:pdsquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PDS Simple Trace Query
Request:
· PDS Simple Trace Query (QUPA_IN000005UK03)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Simple Trace Query Response (QUPA_IN000007UK03)
Note:  Responses to this query will be returned in an XML wrapper called traceQueryResponse.
	Web Service
	RIF Form
	new

	PDS Advanced Trace Query
Request:
· PDS Advanced Trace Query (QUPA_IN000006UK05)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Advanced Trace Query Response (QUPA_IN000011UK05)
	ebXML Unreliable Asynchronous
	RIF Form
	new

	PDS Retrieval Query
Request:
· PDS Retrieval Query (QUPA_IN000008UK05)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Retrieval Query Response (QUPA_IN000009UK06)
Note:  Responses to this query will be returned in an XML wrapper called retrievalQueryResponse.
	Web Service
	RIF Form
	new

	PDS Confirm NHS Number Query
Request:
· PDS Confirm NHS Number Query (QUPA_IN000016UK04)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Confirm NHS Number Query Response (QUPA_IN000010UK04)
Note:  Responses to this query will be returned in an XML wrapper called confirmNHSNumberResponse.
	Web Service
	RIF Form
	new

	PDS Cross Check Trace Query
Request:
· PDS Cross Check Trace Query (QUPA_IN000014UK03)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
· Query Act Failed (QUQI_IN010000UK14)
· PDS Cross Check Trace Query Response (QUPA_IN000015UK04)
Note:  Responses to this query will be returned in an XML wrapper called crossCheckTraceQueryResponse.
	Web Service
	RIF Form
	new


GP Summary
GP Summary interactions all exist under the service urn:nhs:names:services:psis
Version 2.8
Service: urn:nhs:names:services:psis
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Initial GP Summary ( text )
Request:
· Initial GP Summary ( text ) (REPC_IN150015UK05)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	GP Summary
Request:
· GP Summary (REPC_IN150016UK05)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Withdraw Initial GP Summary ( text )
Request:
· Withdraw Initial GP Summary ( text ) (REPC_IN150017UK02)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing

	Withdraw GP Summary
Request:
· Withdraw GP Summary (REPC_IN150018UK02)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	existing


PSIS Query
PSIS Query interactions all exist under the service urn:nhs:names:services:psisquery
Version 1.7
The PSIS Query service will support [MIM4.2.00] for domain version 1.7
Service: urn:nhs:names:services:psisquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PSIS Event List Query Started
Request:
· PSIS Event List Query Started (QUPC_IN180000UK04)
Response:
· PSIS Event List Retrieval (QUPC_IN200000UK04)
· Query Act Failed (QUQI_IN010000UK14)
· Application Acknowledgement (MCCI_IN010000UK13)*
Note:  Responses to this query will be returned in an XML wrapper called eventListQueryResponse.
	Web Service
	RIF Form
	existing

	PSIS Event Query Started
Request:
· PSIS Event Query Started (QUPC_IN190000UK04)
Response:
· PSIS Event Retrieval (QUPC_IN210000UK04)
· Query Act Failed (QUQI_IN010000UK14)
· Application Acknowledgement (MCCI_IN010000UK13)*
Note:  Responses to this query will be returned in an XML wrapper called eventQueryResponse.
	Web Service
	RIF Form
	existing


Version 2.6
Service: urn:nhs:names:services:psisquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PSIS Document List Data Request
Request:
· PSIS Document List Data Request (QUPC_IN160101UK05)
Response:
· PSIS Document List Data Retrieval (QUPC_IN160102UK05)
· Query Act Failed (QUQI_IN010000UK14)
· Application Acknowledgement (MCCI_IN010000UK13)*
	ebXML Unreliable Asynchronous
	RIF Form
	new

	PSIS Document Data Request
Request:
· PSIS Document Data Request (QUPC_IN160103UK05)
Response:
· PSIS Document Data Retrieval (QUPC_IN160104UK05)
· Query Act Failed (QUQI_IN010000UK14)
· Application Acknowledgement (MCCI_IN010000UK13)*
	ebXML Unreliable Asynchronous
	RIF Form
	new


Discharge
Discharge interactions exist under two services urn:nhs:names:services:dis and urn:nhs:names:services:psis
Multiple destination messages controlled by the sender result in a message to PSIS and other messages over the forward channels.
Version 3.6
Service: urn:nhs:names:services:dis
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient Discharge Report
Request:
· Primary Recipient Discharge Report (POCD_IN150001UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Discharge Report
Request:
· Copy Discharge Report (POCD_IN150003UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Nullify Discharge Report
Request:
· Nullify Discharge Report (POCD_IN150004UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


Service: urn:nhs:names:services:psis
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PSIS Discharge Report
Request:
· PSIS Discharge Report (POCD_IN150002UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	new

	Nullify Discharge Report
Request:
· Nullify Discharge Report (POCD_IN150004UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	new


Emergency Department
Emergency Department interactions exist under two services urn:nhs:names:services:ed and urn:nhs:names:services:psis
Multiple destination messages controlled by the sender result in a message to PSIS and other messages over the forward channels.
Version 3.6
Service: urn:nhs:names:services:ed
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient Emergency Department Report
Request:
· Primary Recipient Emergency Department Report (POCD_IN160001UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Emergency Department Report
Request:
· Copy Emergency Department Report (POCD_IN160003UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Nullify Emergency Department Report
Request:
· Nullify Emergency Department Report (POCD_IN160004UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


Service: urn:nhs:names:services:psis
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PSIS Emergency Department Report
Request:
· PSIS Emergency Department Report (POCD_IN160002UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	new

	Nullify Emergency Department Report
Request:
· Nullify Emergency Department Report (POCD_IN160004UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	new


OutPatients
OutPatients interactions exist under two services urn:nhs:names:services:op and urn:nhs:names:services:psis
Multiple destination messages controlled by the sender result in a message to PSIS and other messages over the forward channels.
Version 1.6
Service: urn:nhs:names:services:op
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient Outpatient Report
Request:
· Primary Recipient Outpatient Report (POCD_IN170001UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Outpatient Report
Request:
· Copy Outpatient Report (POCD_IN170003UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Nullify Outpatient Report
Request:
· Nullify Outpatient Report (POCD_IN170004UK06)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


Alerts
Alerts interactions all exist under the service urn:nhs:names:services:alerts
Designed to carry Information Governance (IG) alerts only for the time being. The range of alert types are specified within the MIM / OID Catalogue.
Version 1.1
Service: urn:nhs:names:services:alerts
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Alert Raised
Request:
· Alert Raised (COMT_IN000001UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
  Note: When this interaction is used to send alerts from the Spine to Healthspace response messages MUST NOT be returned.
	ebXML Reliable Asynchronous
	State Exchange Form
	new


CAF (AKA Health and Social Care Integration)
CAF (also known as Health and Social Care Integration) interactions exist under two services urn:nhs:names:services:caf and urn:nhs:names:services:psis
Designed to carry Common Assessment Framework messages.
Version TBD-1.0
Service: urn:nhs:names:services:caf
<<<Section removed>>>

Service: urn:nhs:names:services:psis
<<<Section removed>>>

Version 1.0
Service: urn:nhs:names:services:caf
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient Contact Assessment
Request:
· Primary Recipient Contact Assessment (POCD_IN010001UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Contact Assessment
Request:
· Copy Contact Assessment (POCD_IN010002UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Primary Recipient Overview Assessment
Request:
· Primary Recipient Overview Assessment (POCD_IN010003UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Overview Assessment
Request:
· Copy Overview Assessment (POCD_IN010004UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Primary Recipient Specialist Assessment Outcome
Request:
· Primary Recipient Specialist Assessment Outcome (POCD_IN010005UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Specialist Assessment Outcome
Request:
· Copy Specialist Assessment Outcome (POCD_IN010006UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Primary Recipient Integrated Care and Support Plan
Request:
· Primary Recipient Integrated Care and Support Plan (POCD_IN010007UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Integrated Care and Support Plan
Request:
· Copy Integrated Care and Support Plan (POCD_IN010008UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Primary Recipient Biographic Information
Request:
· Primary Recipient Biographic Information (POCD_IN010009UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Biographic Information
Request:
· Copy Biographic Information (POCD_IN010010UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing


Version 2.1
Service: urn:nhs:names:services:caf
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient Contact Assessment
Request:
· Primary Recipient Contact Assessment (POCD_IN010001UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Contact Assessment
Request:
· Copy Contact Assessment (POCD_IN010002UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Primary Recipient Overview Assessment
Request:
· Primary Recipient Overview Assessment (POCD_IN010003UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Overview Assessment
Request:
· Copy Overview Assessment (POCD_IN010004UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Primary Recipient Specialist Assessment Outcome
Request:
· Primary Recipient Specialist Assessment Outcome (POCD_IN010005UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Specialist Assessment Outcome
Request:
· Copy Specialist Assessment Outcome (POCD_IN010006UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Primary Recipient Integrated Care and Support Plan
Request:
· Primary Recipient Integrated Care and Support Plan (POCD_IN010007UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Integrated Care and Support Plan
Request:
· Copy Integrated Care and Support Plan (POCD_IN010008UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Primary Recipient Biographic Information
Request:
· Primary Recipient Biographic Information (POCD_IN010009UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Copy Biographic Information
Request:
· Copy Biographic Information (POCD_IN010010UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Nullify Health and Social Care Integration Document
Request:
· Nullify Health and Social Care Integration Document (POCD_IN010011UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing


Version 2.2

Service: urn:nhs:names:services:caf

	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Assessment Notification Under Section 2 of the Community Care (Delayed Discharge) Act 2003

Request:
· Assessment Notification Under Section 2 of the Community Care (Delayed Discharge) Act 2003

·  (COMT_IN010020UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Assessment Notification Accept Response Under Section 2 of the Community Care (Delayed Discharge) Act 2003 Authenticated User Session

Request:
· Assessment Notification Accept Response Under Section 2 of the Community Care (Delayed Discharge) Act 2003 Authenticated User Session

· (COMT_IN010021UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Assessment Notification Reject Response Under Section 2 of the Community Care (Delayed Discharge) Act 2003 Authenticated User Session 

Request:
· Assessment Notification Reject Response Under Section 2 of the Community Care (Delayed Discharge) Act 2003 Authenticated User Session 

· (COMT_IN010022UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Assessment Notification Accept Response Under Section 2 of the Community Care (Delayed Discharge) Act 2003 Unattended Messaging Session 

Request:
· Assessment Notification Accept Response Under Section 2 of the Community Care (Delayed Discharge) Act 2003 Unattended Messaging Session 

· (COMT_IN010023UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Assessment Notification Reject Response Under Section 2 of the Community Care (Delayed Discharge) Act 2003 Unattended Messaging Session 

Request:
· Assessment Notification Reject Response Under Section 2 of the Community Care (Delayed Discharge) Act 2003 Unattended Messaging Session 

· (COMT_IN010024UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Discharge Notification Under Section 5 of the Community Care (Delayed Discharge) Act 2003 

Request:
· Discharge Notification Under Section 5 of the Community Care (Delayed Discharge) Act 2003 

· (COMT_IN010025UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Discharge Notification Agreement Under Section 5 of the Community Care (Delayed Discharge) Act 2003 Authenticated User Session Agreement 

Request:
· Discharge Notification Agreement Under Section 5 of the Community Care (Delayed Discharge) Act 2003 Authenticated User Session Agreement 

· (COMT_IN010026UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Discharge Notification Disagreement Under Section 5 of the Community Care (Delayed Discharge) Act 2003 Authenticated User Session 

Request:
· Discharge Notification Disagreement Under Section 5 of the Community Care (Delayed Discharge) Act 2003 Authenticated User Session 

· (COMT_IN010027UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Discharge Notification Agreement Under Section 5 of the Community Care (Delayed Discharge) Act 2003 Unattended Messaging Session 

Request:
· Discharge Notification Agreement Under Section 5 of the Community Care (Delayed Discharge) Act 2003 Unattended Messaging Session 

· (COMT_IN010028UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Discharge Notification Disagreement Under Section 5 of the Community Care (Delayed Discharge) Act 2003 Unattended Messaging Session

Request:
· Discharge Notification Disagreement Under Section 5 of the Community Care (Delayed Discharge) Act 2003 Unattended Messaging Session

·  (COMT_IN010029UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Cancellation Notification Under Section 3 of the Community Care (Delayed Discharge) Act 2003 (Discharge Notification Only) 

Request:
· Cancellation Notification Under Section 3 of the Community Care (Delayed Discharge) Act 2003 (Discharge Notification Only) 

· (COMT_IN010030UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Cancellation Notification Under Section 3 of the Community Care (Delayed Discharge) Act 2003 (Complete Process) 

Request:
· Cancellation Notification Under Section 3 of the Community Care (Delayed Discharge) Act 2003 (Complete Process) 

· (COMT_IN010031UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary NHS Continuing Healthcare Checklist Assessment 

Request:
· Primary NHS Continuing Healthcare Checklist Assessment 

· (POCD_IN011001GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary Recipient Contact Assessment
Request:
· Primary NHS Continuing Healthcare Checklist Assessment Accept Response 

· (COMT_IN011002GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary NHS Continuing Healthcare Checklist Assessment Reject Response 

Request:
· Primary NHS Continuing Healthcare Checklist Assessment Reject Response 

· (COMT_IN011003GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Cancellation of Continuing Healthcare Process Notification 

Request:
· Cancellation of Continuing Healthcare Process Notification 

· (COMT_IN011004GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary NHS Continuing Healthcare Fast Track Pathway Tool 

Request:
· Primary NHS Continuing Healthcare Fast Track Pathway Tool 

· (POCD_IN011005GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary NHS Continuing Healthcare Fast Track Pathway Tool Accept Response 

Request:
· Primary NHS Continuing Healthcare Fast Track Pathway Tool Accept Response 

· (COMT_IN011006GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary NHS Continuing Healthcare Fast Track Pathway Tool Reject Response 

Request:
· Primary NHS Continuing Healthcare Fast Track Pathway Tool Reject Response 

· (COMT_IN011007GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary NHS Continuing Healthcare Request For Assessment 

Request:
· Primary NHS Continuing Healthcare Request For Assessment 

· (POCD_IN011011GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary NHS Continuing Healthcare Request For Assessment Accept Response 

Request:
· Primary NHS Continuing Healthcare Request For Assessment Accept Response 

· (COMT_IN011012GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary NHS Continuing Healthcare Request For Assessment Reject Response 

Request:
· Primary NHS Continuing Healthcare Request For Assessment Reject Response 

· (COMT_IN011013GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Nullify Health and Social Care Integration Document 

Request:
· Nullify Health and Social Care Integration Document 

· (POCD_IN010014GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


Pathology (AKA Laboratory)
Pathology interactions all exist under the service urn:nhs:names:services:pat
Designed to forward Pathology messages.
Version 1.1
Service: urn:nhs:names:services:pat
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Order Fulfillment Request with Receiver ResponsibilitiesRequest:

· Order Fulfillment Request with Receiver Responsibilities (POLB_IN020001UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	Existing

	Order Confirm Response

Request:
· Order Confirm Response (POLB_IN020002UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	Existing

	Order Reject Response

Request:
· Order Reject Response (POLB_IN020003UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Result In Progress with Receiver Responsibilities

Request:
· Result In Progress with Receiver Responsibilities (POLB_IN020005UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Result Complete with Receiver Responsibilities

Request:
· Result Complete with Receiver Responsibilities (POLB_IN020006UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing

	Result Event State Change with Receiver Responsibilities

Request:
· Result Event State Change with Receiver Responsibilities (POLB_IN020010UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)*
	Multi-hop Intermediary Reliability
	State Exchange Form
	existing


Common Content (also known as Common Point to Point)
Common Content (also known as Common Point to Point) interactions all exist under the service urn:nhs:names:services:cc
Designed to forward unspecified messages.
Version 1.1
Service: urn:nhs:names:services:cc
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Common Content Forward Reliable
Request:
· Common Content Forward Reliable (COPC_IN000001UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


Admissions
Admissions interactions exist under two services urn:nhs:names:services:adm and urn:nhs:names:services:psis
Designed to carry Admissions Report messages.
Version 1.1
Service: urn:nhs:names:services:adm
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient Admissions Report
Request:
· Primary Recipient Admissions Report (POCD_IN040001UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Admissions Report
Request:
· Copy Admissions Report (POCD_IN040003UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Nullify Admissions Report
Request:
· Nullify Admissions Report (POCD_IN040004UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


Service: urn:nhs:names:services:psis
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Nullify Admissions Report
Request:
· Nullify Admissions Report (POCD_IN040004UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	new


Out of Hours
Out of Hours interactions exist under two services urn:nhs:names:services:ooh and urn:nhs:names:services:psis
Designed to carry Out of Hours Report messages.
Version 1.0
Service: urn:nhs:names:services:ooh
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient Out of Hours Report
Request:
· Primary Recipient Out of Hours Report (POCD_IN020001UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Out of Hours Report
Request:
· Copy Out of Hours Report (POCD_IN020003UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Nullify Out of Hours Report
Request:
· Nullify Out of Hours Report (POCD_IN020004UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


Ambulance Messaging
Ambulance Messaging interactions exist under two services urn:nhs:names:services:am and urn:nhs:names:services:psis
Designed to carry Ambulance Service Patient Report messages.
Version 1.1
Service: urn:nhs:names:services:am
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Nullify Ambulance Service Patient Report
Request:
· Nullify Ambulance Service Patient Report (POCD_IN030004UK01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	new


Service: urn:nhs:names:services:psis
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	PSIS Ambulance Service Patient Report
Request:
· PSIS Ambulance Service Patient Report (POCD_IN030002UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	new

	Nullify Ambulance Service Patient Report
Request:
· Nullify Ambulance Service Patient Report (POCD_IN030004UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	ebXML Reliable Asynchronous
	State Exchange Form
	new


Mental Health
Mental Health interactions exist under two services urn:nhs:names:services:mh and urn:nhs:names:services:psis
Designed to carry Mental Health Report messages.
Version 1.0
Service: urn:nhs:names:services:mh
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient Mental Health and Substance Misuse Specialist Services Entry Assessment
Request:
· Primary Recipient Mental Health and Substance Misuse Specialist Services Entry Assessment (POCD_IN060001UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary Recipient Mental Health and Substance Misuse Specialist Services Update Assessment
Request:
· Primary Recipient Mental Health and Substance Misuse Specialist Services Update Assessment (POCD_IN060002UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary Recipient Mental Health and Substance Misuse Specialist Services Exit Assessment
Request:
· Primary Recipient Mental Health and Substance Misuse Specialist Services Exit Assessment (POCD_IN060003UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary Recipient Mental Health and Substance Misuse Specialist Services Care Record Summary (CPA)
Request:
· Primary Recipient Mental Health and Substance Misuse Specialist Services Care Record Summary (CPA) (POCD_IN060004UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary Recipient Mental Health and Substance Misuse Specialist Services Care Record Summary (MOC)
Request:
· Primary Recipient Mental Health and Substance Misuse Specialist Services Care Record Summary (MOC) (POCD_IN060005UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary Recipient Mental Health and Substance Misuse Specialist Services Care Record Summary (Unspecified)
Request:
· Primary Recipient Mental Health and Substance Misuse Specialist Services Care Record Summary (Unspecified) (POCD_IN060006UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary Recipient Mental Health Act Assessment Outside Specialist Services
Request:
· Primary Recipient Mental Health Act Assessment Outside Specialist Services (POCD_IN060007UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Primary Recipient Mental Health Act Assessment Update Non-Specialist Services
Request:
· Primary Recipient Mental Health Act Assessment Update Non-Specialist Services (POCD_IN060008UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Mental Health and Substance Misuse Specialist Services Entry Assessment
Request:
· Copy Mental Health and Substance Misuse Specialist Services Entry Assessment (POCD_IN060017UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Mental Health and Substance Misuse Specialist Services Update Assessment
Request:
· Copy Mental Health and Substance Misuse Specialist Services Update Assessment (POCD_IN060018UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Mental Health and Substance Misuse Specialist Services Exit Assessment
Request:
· Copy Mental Health and Substance Misuse Specialist Services Exit Assessment (POCD_IN060019UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Mental Health and Substance Misuse Specialist Services Care Record Summary (CPA)
Request:
· Copy Mental Health and Substance Misuse Specialist Services Care Record Summary (CPA) (POCD_IN060020UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Mental Health and Substance Misuse Specialist Services Care Record Summary (MOC)
Request:
· Copy Mental Health and Substance Misuse Specialist Services Care Record Summary (MOC) (POCD_IN060021UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Mental Health and Substance Misuse Specialist Services Care Record Summary (Unspecified)
Request:
· Copy Mental Health and Substance Misuse Specialist Services Care Record Summary (Unspecified) (POCD_IN060022UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Mental Health Act Assessment Outside Specialist Services
Request:
· Copy Mental Health Act Assessment Outside Specialist Services (POCD_IN060023UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Mental Health Act Assessment Update Non-Specialist Services
Request:
· Copy Mental Health Act Assessment Update Non-Specialist Services (POCD_IN060024UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Nullify Mental Health Document
Request:
· Nullify Mental Health Document (POCD_IN010025UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


NHS Direct
NHS Direct interactions exist under two services urn:nhs:names:services:nd and urn:nhs:names:services:psis
Designed to carry NHS Direct Report messages.
Version 1.0
Service: urn:nhs:names:services:nd
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient NHS Direct Report
Request:
· Primary Recipient NHS Direct Report (POCD_IN070001UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy NHS Direct Report
Request:
· Copy NHS Direct Report (POCD_IN070003UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Nullify NHS Direct Report
Request:
· Nullify NHS Direct Report (POCD_IN070004UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


Diagnostic Imaging Reporting
Diagnostic Imaging Reporting interactions exist under two services urn:nhs:names:services:dir and urn:nhs:names:services:psis
Designed to carry Diagnostic Imaging Report messages.
Version 1.0
Service: urn:nhs:names:services:dir
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	Primary Recipient Diagnostic Imaging Report
Request:
· Primary Recipient Diagnostic Imaging Report (POCD_IN050001UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Copy Diagnostic Imaging Report
Request:
· Copy Diagnostic Imaging Report (POCD_IN050003UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new

	Nullify Diagnostic Imaging Report 
Request:
· Nullify Diagnostic Imaging Report (POCD_IN050004UK01)
Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	new


General Practice Extraction Service

General Practice Extraction Service interactions exist under the GPES service:-  urn:nhs:names:services:gpes 

Designed to carry General Practice Extraction Service messages.

Version 1.0

Service: urn:nhs:names:services:gpes

	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	GPES-Q-E Run Time Parameter Request

Request:
· GPES-Q-E Run Time Parameter Request (COMT_IN000001GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	New

	GPES-E-Q Run Time Parameter Response

Request:
· GPES-E-Q Run Time Parameter Response (COMT_IN000002GB01)

Response:
Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	New

	GPES-Q-E Notification

Request:
· GPES-Q-E Notification (COMT_IN000003GB01)

Response:
Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	New

	GPES-E-Q Notification

Request:
· GPES-E-Q Notification (COMT_IN000004GB01)

Response:
Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	New

	GPES-E-Q Query Results

Request:
· GPES-E-Q Query Results (COMT_IN000005GB01)

Response:
Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	New

	GPES-Q-E Query Results Acknowledgement

Request:
· GPES-Q-E Query Results Acknowledgement (COMT_IN000006GB01)

Response:
Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	New

	GPES-APP-ACK
Request:
Application Acknowledgement (MCCI_IN010000UK13)
Response:
Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	New


Inter-operability Toolkit Service

Inter-operability Toolkit interactions exist under the ITK service:-  urn:nhs:names:services:itk 

Designed for trunking of ITK messages over SPINE.

Version 1.0

Service: urn:nhs:names:services:itk

	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	ITK Trunk Message (MHR)

Request:
· ITK Trunk Message (MHR) (COPC_IN000002GB01)

Response:
· Application Acknowledgement (MCCI_IN010000UK13)
	Multi-hop Intermediary Reliability
	State Exchange Form
	New

	ITK Trunk Message (EPR)

Request:
· ITK Trunk Message (EPR) (COPC_IN000001GB01)

Response:
Application Acknowledgement (MCCI_IN010000UK13)
	Multi-Hop End-Party Reliability
	N/A
	New


Child Protection Information Service
CPIS interactions exist under the CPIS service:-  urn:nhs:names:services:cpis 
Version 1.0

Service: urn:nhs:names:services:cpisquery
	Interaction (Business Action)
	MHS Message Behaviour (from [BT-Mbeh])
	HL7 Forms of Operation (from Part 2 of this specification)
	Status

	ITK Trunk Message (MHR)

Request:
· CPIS Query (QUPC_IN000006GB01)

Response:
· CPIS Response (REPC_IN000007GB01)

· Query Act Failed (QUQI_IN010000UK14)
· Application Acknowledgement (MCCI_IN010000UK13)*

Note:  Responses to this query will be returned in an XML wrapper called NHSQueryResponse

	Web Service
	RIF Form
	new


	Copyright © 2018, NHS Digital
	         
	Page 6 of 76



